# Information about the inernals of the PC14xx Sharp PC family

Compiled from several sources

## The LCD Screen

Character location in memory:

| 1-6 | 12288 (&3000) - 12317 |
| --- | --- |
| 7-9 | 12333 - 12347 |
| 10-12 | 12318 - 12332 |
| 13-15 | 12396 (&306C) - 12382 |
| 16-18 | 12411 - 12397 |
| 19-24 | 12381 - 12352 |

LCD Display memory map

| Display flags |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 12348 | 001 カナ kana (Katakana) 002 ⼩ klein (Kanji) 004 SML 008 ▃ STAT 016 ▃ MATRIX 032 ▃ (rechts von PRO) 064 ▃ (links von CAL) | 12349 | 001 BUSY 002 DEF 004 SHIFT 008 HYP 016 ▃ PRO 032 ▃ RUN 064 ▃ CAL | 12412 | 001 E 002 M 004 ( ) 008 RAD 016 G 032 DE 064 ▃ PRINT |

Example of calculating the value of a display column:

Display flag bits

| 12348 | 12349 | 12412 |
| --- | --- | --- |
| 1 | lf (weird chars) | BUSY | E |
| 2 | m (weird chars) | DEF | M |
| 4 | SML | SHIFT | () |
| 8 | STAT | HYP | RAD |
| 16 | MATRIX | PRO | G |
| 32 | BASIC | RUN | DE |
| 64 | Block | CAL | PRINT |

### PC-1403 ROM routines

04B4 - to turn the LCD off:

CALL &04B4

04B8 - to turn the LCD on:

CALL &04B8

Note how the LCD on/off is controlled by bit-0 of the CPU C-port, which includes other control bits.

Below the 04B4 and 04B8 disassembly:

LIA		0   	; LCD_OFF

JRP		SET_C

LIA		1   	; LCD_ON

SET_C:	LIP		5F  	; OutC Port

EXAM

OUTC

RTN

4587 (ROM bank #9) - Display an ASCII string.

The string is assumed to be stored at locations starting at 0x10 (internal CPU RAM; see below) for max 24 char and terminated by a 0. Note how no boundary safety is present, so a crash could happen is the string is not terminated within the 24 characters.

A ROM-bank switch to bank #9 is needed, to access the routine.

Entry with J = 1.

Below an example code fragment using the 4587 routine:

LIJ  1
LIDP 0x3C00	; Read current memory bank#
LDD
PUSH		; save it to stack
LIA  0x09	; select External ROM bank #9
STD
CALL 0x4587	; PC-1403 ROM entry point for the print
POP	 	; Restore original bank#
LIDP 0x3C00	; bankswitch
STD		; and set it

4CED (ROM bank #9) – move PRINT buffer to internal RAM

Clear internal RAM at 0x10 and transfer here 24 characters from FD60 (print buffer, possibly used by BASIC too).

Content of FF6C/FF6D should prepared with the pointer to string end.

Entry with J = 1.

A ROM-bank switch to bank #9 is needed, to access the routine.

## BASIC program memory

- The program memory starts at 32768 and ends at 65535. The rest belongs to the system like display, interfaces and main program!

- The first BASIC line starts at 32817 (&8031). You can move this position and move it back to have space for data. The memory locations 65281 (&FF01) and 65282 (&FF02) define this position:

PEEK 65281+256* PEEK 65282 = start address.

On a fresh PC-1403 (8K RAM model; not ‘H’): 48 + 224*256 = 57392 = $E030

- At this address there must be a byte with the value 255. After this, the first line starts.

- The end of the BASIC program is marked with a 255, too. This address is given in the pointer at 65283 and 65284. Address of this end position = PEEK 65283+ 256* PEEK 65284

In order to create room for custom assembler program, or data, POKE the new BASIC start address in 65281 (&FF01) and 65282 (&FF02) and then reset program area with a NEW command.

E.g. Reserving 2Kb (2048 bytes) on top of the program area (PC-1403):

NEW
MEM
> 6878
POKE &FF01, 48, (224 + 2048/256)
NEW
MEM
> 4830

Handwritten notes by HrastProgrammer:

## Some memory pointers

| &0000 - &1FFF | 0 - 8191 | Internes ROM |
| --- | --- | --- |
| &E030 | 57392 | Beginn Basic (standard) |
| &FB10 - &FBDF | 64272 - 64479 | Standardvariable |
| &3000 - &301D | 12288 - 12317 | Display Stellen 1-6 |
| &301E - &302C | 12318 - 12332 | Display Stellen 10-12 |
| &302D - &303B | 12333 - 12347 | Display Stellen 7-9 |
| &3040 - &305D | 12352 - 12381 | Display Stellen 24-19 |
| &305E - &306C | 12382 - 12396 | Display Stellen 15-13 |
| &306D - &307B | 12397 - 12411 | Display Stellen 18-16 |
| &303C | 12348 | Display Statusanzeigen |
| &303D | 12349 | Display Statusanzeigen |
| &307C | 12412 | Display Statusanzeigen |
| &FF09 - &FF10 | 65289 - 65296 | Passwort |
| &FF47 - &FF48 | 65351 - 65352 | LoByte WAIT Intervall HiByte |

| Address Range (Hex) | Description | Translation/Notes |
| --- | --- | --- |
| 12348, 12349, 12412 | DISPLAY INDICATORS | Memory addresses for display status indicators |
| 12440 - 12447 | SEED FOR RND | Random number generator seed location |
| 57350 - 57391 | Copy to ibew space | Copies to memory area (65280 - 65321) |
| 64576 - 64587 | Storing result of det X→x operation | Saves operation results |
| 64592 - 64767 | Most likely for disk operations | 176 bytes free |
| 64864 - 64887 | Text displayed on screen | At a specific screen position |
| 65120 - 65199 | STRING WORKSPACE | Memory area for string operations |
| 65200 - 65279 | INPUT BUFFER | Buffer for user input |
| 65289 - 65296 | PASS STRING | (65289) = 245 (8F5) |
| 65326, 65327 | Variable address | Address of variable last used in mode/ADET... (-1) |
| 65488 - 65519 | Copy notespage enabling | 32 bytes free |
| 65512 - 65519 | CALCULATOR MEMORY | Memory area for calculations |
| 65520 - 65527 | LAST CALCULATOR | Previous calculation storage |
| 65528 - 65535 | ANSWER | Special place for result decimal point |
| 65287 – 65288 | Begin of arrays’ area |  |

## BASIC command tokens

| Codetabelle |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | &8x | &9x | &Ax | &Bx | &Cx | &Dx | &Ex |
| &x0 | MDF | FACT | RND | RUN | RANDOM | TO | GOSUB |
| &x1 | REC | LN | AND | NEW | DEGREE | STEP | AREAD |
| &x2 | POL | LOG | OR | CONT | RADIAN | THEN | LPRINT |
| &x3 | ROT | EXP | NOT | PASS | GRAD | ON | RETURN |
| &x4 | DECI | SQR | ASC | LIST | BEEP | IF | RESTORE |
| &x5 | HEX | SIN | VAL | LLIST | WAIT | FOR | CHAIN |
| &x6 | TEN | COS | LEN | CSAVE | GOTO | LET |  |
| &x7 | RCP | TAN | PEEK | CLOAD | TRON | REM |  |
| &x8 | SQU | INT | CHR$ | MERGE | TROFF | END |  |
| &x9 | CUR | ABS | STR$ |  | CLEAR | NEXT |  |
| &xA | HSN | SGN | MID$ |  | USING | STOP |  |
| &xB | HCS | DEG | LEFT$ | OPEN | DIM | READ |  |
| &xC | HTN | DMS | RIGHT$ | CLOSE | CALL | DATA |  |
| &xD | AHS | ASN | INKEY$ | SAVE | POKE | PAUSE |  |
| &xE | AHC | ACS | PI | LOAD |  | PRINT |  |
| &xF | AHT | ATN | MEM |  |  | INPUT | &EF,xx |

Extended (2-bytes &EF,xx) BASIC command tokens

| &EF,xx |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | &EF,&8x | &EF,&Ax | &EF,&Bx | &EF,&Ex |
| &EF,&x0 | EOF |  | FILES | OUTPUT |
| &EF,&x1 | DSKF | XOR | LFILES | APPEND |
| &EF,&x2 | LOF |  | INIT | AS |
| &EF,&x3 | LOC |  | KILL |  |
| &EF,&x4 |  |  | NAME |  |
| &EF,&x5 |  |  | SET |  |
| &EF,&x6 |  |  | COPY |  |
| &EF,&x8 |  | RENUM |  |  |
| &EF,&x9 |  | DELETE |  |  |

## Standard variable locations

| Numerisch |  | Text |  | Hex | Dez |
| --- | --- | --- | --- | --- | --- |
| A | A(1) | A$ | A$(1) | &FBD8 - &FBDF | 64472 - 64479 |
| B | A(2) | B$ | A$(2) | &FBD0 - &FBD7 | 64464 - 64471 |
| C | A(3) | C$ | A$(3) | &FBC8 - &FBCF | 64456 - 64463 |
| D | A(4) | D$ | A$(4) | &FBC0 - &FBC7 | 64448 - 64455 |
| E | A(5) | E$ | A$(5) | &FBB8 - &FBBF | 64440 - 64447 |
| F | A(6) | F$ | A$(6) | &FBB0 - &FBB7 | 64432 - 64439 |
| G | A(7) | G$ | A$(7) | &FBA8 - &FBAF | 64424 - 64431 |
| H | A(8) | H$ | A$(8) | &FBA0 - &FBA7 | 64416 - 64423 |
| I | A(9) | I$ | A$(9) | &FB98 - &FB9F | 64408 - 64415 |
| J | A(10) | J$ | A$(10) | &FB90 - &FB97 | 64400 - 64407 |
| K | A(11) | K$ | A$(11) | &FB88 - &FB8F | 64392 - 64399 |
| L | A(12) | L$ | A$(12) | &FB80 - &FB87 | 64384 - 64391 |
| M | A(13) | M$ | A$(13) | &FB78 - &FB7F | 64376 - 64383 |
| N | A(14) | N$ | A$(14) | &FB70 - &FB77 | 64368 - 64375 |
| O | A(15) | O$ | A$(15) | &FB68 - &FB6F | 64360 - 64367 |
| P | A(16) | P$ | A$(16) | &FB60 - &FB67 | 64352 - 64359 |
| Q | A(17) | Q$ | A$(17) | &FB58 - &FB5F | 64344 - 64351 |
| R | A(18) | R$ | A$(18) | &FB50 - &FB57 | 64336 - 64343 |
| S | A(19) | S$ | A$(19) | &FB48 - &FB4F | 64328 - 64335 |
| T | A(20) | T$ | A$(20) | &FB40 - &FB47 | 64320 - 64327 |
| U | A(21) | U$ | A$(21) | &FB38 - &FB3F | 64312 - 64319 |
| V | A(22) | V$ | A$(22) | &FB30 - &FB37 | 64304 - 64311 |
| W | A(23) | W$ | A$(23) | &FB28 - &FB2F | 64296 - 64303 |
| X | A(24) | X$ | A$(24) | &FB20 - &FB27 | 64288 - 64295 |
| Y | A(25) | Y$ | A$(25) | &FB18 - &FB1F | 64280 - 64287 |
| Z | A(26) | Z$ | A$(26) | &FB10 - &FB17 | 64272 - 64279 |

## Keyboard

The PC1403(H) keyboard is scanned using the IA port and the external K port, having the following layout:

Here’s a fragment from the ROM routine scanning the IA port, as an example:

LIP	5C	; IA port address

LIA	01	; initial strobe mask

EXAM

OUTA		; strobe on IA1

LIDP	3E00

ANID	00	; clear K-port strobe

WAIT	25

LDM		; IA port -> A

RC

SL

EXAM		; A -> IA

LP	0A

INA		; get a key (if any)

Note how the IA is bi-directional, used both for output to set the strobe signals, and input to get the pressed key (if any), while the K-port is for output only, to set strobes only.

### PC 1403 ROM routines

1494 - Wait for a keystroke.
Returns with keycode (see below) in A registry:

CALL &1494

0E68 - Scan the keyboard (non-blocking)
Returns with keycode (see below) in A registry. 0xFF if none has been pressed. 
Carry flag set if BRK pressed:

CALL &0E68

NOTE – I haven’t figured out yet why calling repeatedly &0E68 in a loop seems like spinning the CPU too much, apparently drawing current and dimming the LCD, while a call to &1494 (while still using the 0E68 inside) keeps the LCD display on and clear, at the same time being responsive enough to key strokes.

### Key codes

List of keycodes returned by 1494 or 0E68 routines:

//       SHARP  POCKET COMPUTER PC-1403H  32KB                 +---+ +---+ +---+ +---+ +---+ +---+

//                                                             |hyp| |sin| |cos| |tan| |FSE| |CCE|

//  on                                                         +---+ +---+ +---+ +---+ +---+ +---+

// +-+ +----------------------------------------------------+    67    66    65    64    4     6

// ¦ ¦ ¦                                    DEG             |  +---+ +---+ +---+ +---+ +---+ +---+

// +-+ ¦ >                                                  |  |HEX| |DEG| | ln| |log| |1/x| | ^ |

// off ¦      _                                             |  +---+ +---+ +---+ +---+ +---+ +---+

//     +----------------------------------------------------+    75    74    73    72    11    5

//       CAL RUN PRO                                           +---+ +---+ +---+ +---+ +---+ +---+

//                                                             |EXP| |y^x| | v~| |x^2| | ( | | ) |

//                                                             +---+ +---+ +---+ +---+ +---+ +---+

//                                                               83    82    81    80    17    12

// +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+  +---+  +---+  +---+  +---+  +---+

// |CAL| |BAS| |BRK| |DEF| | v | | ^ | | < | | > | |SML| |SHI|  | 7 |  | 8 |  | 9 |  | / |  |X>M|

// +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+  +---+  +---+  +---+  +---+  +---+

//   25    22    -     37    3     10    16    21    36    38     35     34     33     32     31

// +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+  +---+  +---+  +---+  +---+  +---+

// | Q | | W | | E | | R | | T | | Y | | U | | I | | O | | P |  | 4 |  | 5 |  | 6 |  | * |  | RM|

// +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+  +---+  +---+  +---+  +---+  +---+

//   46    54    62    70    78     2     9    15    20    24     43     42     41     40     39

// +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+  +---+  +---+  +---+  +---+  +---+

// | A | | S | | D | | F | | G | | H | | J | | K | | L | | , |  | 1 |  | 2 |  | 3 |  | - |  | M+|

// +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+  +---+  +---+  +---+  +---+  +---+

//   45    53    61    69    77     1     8    14    19    23     51     50     49     48     47

// +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---------+  +---+  +---+  +---+  +---+  +---+

// | Z | | X | | C | | V | | B | | N | | M | |SPC| |  ENTER  |  | 0 |  |+/-|  | . |  | + |  | = |

// +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---+ +---------+  +---+  +---+  +---+  +---+  +---+

//   44    52    60    68    76     0     7    13       18        59     58     57     56     55

### ROM Routines for other PC models (different from 1403(H))

(excerpt from “das_grosse_pocket_computer_buch-compressed.pdf”; addresses in decimal)

GETKEY

PC 140X: 5208        PC 1421: 5153
PC 126X: 4414        PC 1350: 4618

Function:

Waits until a key is pressed, then returns with the key code in A and 24634 (PC 126X: 26156, PC 1350: 28503)

Notes:

The SHIFT key assignments are also taken into account. If the OFF switch is pressed, the routine returns with A=10. No automatic shutdown of the computer if no key is pressed. The keys X+>M, RM, M+, +/-, F<->E are not recognized on the PC 14XX.

When calling the program in RUN mode, the program will be terminated with ERROR 4 if the key combination 'DEF letter key' is pressed.

INKEY

PC-140X: 3387        PC-1421: 3359
PC-126X: 2729        PC-1350: 3003

Function:

If a key is pressed, the code of this key is stored in memory cell Y+1 of the external RAM. So first load the Y register with a suitable address. When you then call the program INKEY and a key is pressed, the code of this key is stored in memory cell Y+1 with the command IYS.

## The SC-61860 CPU

### Block diagram

(as from PC-1250 service manual)

### Mnemonics

|  | 0x | 1x |  | 2x | 3x | 4x | 5x | 6x | 7x | 8x-Bx | Cx | Dx | Ex-Fx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x0 | LII n | LIDP nm |  | LDP | STP | INCI | INCP | ANIM n | ADIM n | LP l | INCJ | SC | CAL ln |
| x1 | LIJ n | LIDL n |  | LDQ | STQ | DECI | DECP | ORIM n | SBIM n |  | DECJ | RC |  |
| x2 | LIA n | LIP n |  | LDR | STR | INCA | STD | TSIM n | RZ n |  | INCB | SR |  |
| x3 | LIB n | LIQ n |  | CLRA | NOPT | DECA | MVDM | CPIM n | RZ n |  | DECB | NOPW |  |
| x4 | IX | ADB |  | IXL | PUSH | ADM | MVMP | ANIA n | ADIA n |  | ADCM | ANID n |  |
| x5 | DX | SBB |  | DXL | MVWP | SBM | MVMD | ORIA n | SBIA n |  | SBCM | ORID n |  |
| x6 | IY |  |  | IYS |  | ANMA | LDPC | TSIA n | RZ n |  | TSMA | TSID n |  |
| x7 | DY |  |  | DYS | RTN | ORMA | LDD | CPIA n | RZ n |  | CPMA | SZ n |  |
| x8 | MVW | MVWD |  | JRNZP n | JRZP n | INCK | SWP | NOPT | CALL nm |  | INCL | LEAVE |  |
| x9 | EXW | EXWD |  | JRNZM n | JRZM n | DECK | LDM | CASE | JP nm |  | DECL | NOPW |  |
| xA | MVB | MVBD |  | JRNCP n | JRCP n | INCM | SL | NOPT | SET knm |  | INCN | EXAB |  |
| xB | EXB | EXBD |  | JRNCM n | JRCM n | DECM | POP | TEST n |  |  | DECN | EXAM |  |
| xC | ADN | SRW |  | JRP n |  | INA |  |  | JPNZ nm |  | INB |  |  |
| xD | SBN | SLW |  | JRM n |  | NOPW | OUTA |  | JPNC nm |  | NOPW | OUTB |  |
| xE | ADW | FILM |  |  |  | WAIT n |  |  | JPZ nm |  | NOPT |  |  |
| xF | SBW | FILD |  | LOOP n |  | IPXL | OUTF | IPXH | JPC nm |  |  | OUTC |  |
| k,n,m = 00-FF |  |  |
| l = 00-3F |  |  |
| ln = 0000-1FFF |  |  |

Register RAM

| 00 | I |
| --- | --- |
| 01 | J |
| 02 | A |
| 03 | B |
| 04 | XL |
| 05 | XH |
| 06 | YL |
| 07 | YH |
| 08 | K |
| 09 | L |
| 0A | M |
| 0B | N |
| 0C |  |
| … |  |
| 10 | X float |
| … |  |
| 17 |  |
| 18 | Y float |
| … |  |
| 1F |  |
| … | … |
| . . . | Stack |
| 5B |  |
| 5C | IA |
| 5D | IB |
| 5E | FO |
| 5F | Cout |

## The floating point representation

(image above: credit HrastProgrammer’s notes)

From the PC 1350 Machine Language manual:

The internal representation of the floating point, used in BASIC variables too, is made of 8 bytes: a numeric value consists of an exponent, a mantissa sign, and a mantissa. 2 of them for the exponent, including sign, and 6 for the mantissa, in BCD format (i.e. a 4-bit nibble for each 0-9 digit).

Numbers from -9.999999999 x 1099 to 9.999999999 x 1099 can be represented.

Exponent

The exponent is represented using two decimal digits.

The most significant digit is always zero for positive numbers.

Negative exponents are represented using a complement: 901 (10-99) to 099 (1099):

Mantissa sign

Zero (0) is used when the mantissa is positive.

Eight (8) is used when the mantissa is negative.

Example numbers stored at BASIC variable ‘B’ location:

6CF0H	6CF7H

| 00H | 30H | 15H | 00H | 00H | 00H | 00H | 00H | 1500 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00H | 00H | 12H | 34H | 56H | 00H | 00H | 00H | 1.23456 |
| 99H | 70H | 12H | 34H | 56H | 78H | 90H | 00H | 0.00123456789 |
| 00H | 88H | 12H | 34H | 00H | 00H | 00H | 00H | -1.234x108 |

## PC-1403 (H) ROM math routines

### Math on register RAM

(credit: HrastProgrammer)

Some of the math routines (hex addresses) in internal ROM (default ROM bank #0), referring to X-Float (0x10..0x17) and Y-float (0x18..0x1F) in the CPU register memory area:

| 0143 | Zero moved to floatY | 0 -> floatY |
| --- | --- | --- |
| 0149 | Zero moved to floatX | 0 -> floatX |
| 0169 | 1 to floatY | 1 -> floatY |
| 10BE | Add | floatX + floatY -> floatX |
| 10D3 | Subtract | floatX – floatY -> floatX |
| 10DF | Multiply | floatX * floatY -> floatX |
| 10E8 | Divide | floatX / floatY -> floatX |
| 1446 | Unsigned 16-bit binary to Float | (19H lsb, 18H msb) -> floatX |
| 144D | Signed 16-bit binary to Float | (19H lsb, 18H msb) -> floatX |
| 1609 | Float to Signed 16-bit binary | floatX -> (19H lsb, 18H msb) |
| 0398 | 16-bit BA moved to Y | B:A -> floatY |
| 039D | 16-bit BA moved to X | B:A -> floatX |
| ... |  |  |

Many more are listed in the handwritten notes by HrastProgrammer, a Slovenian guy who made a hard work reverse-engineering parts of the PC-1403 ROM (back then when no sophisticated debugging tools were available), and who shared them with me some time ago, but they’re hard to decypher… Will add more, as soon as I get to understand them.

### Matrix operations

(credit: HrastProgrammer)

Matrix operations are normally used in PC-1403 while in ‘CAL’ mode. There’s a way to make use of them in BASIC programs too.

The following are the ROM routines (hex addresses) operating on matrices stored as X, Y, and M BASIC matrix variables:

| 65FD | X + Y → X |
| --- | --- |
| 65F7 | X – Y → X |
| 65F7 | X * Y → X |
| 65FC | X * inv Y → X |
| 6604 | inv X → X |
| 660C | x + X → X |
| 6614 | x – X → x |
| 6619 | x * X → X |
| 661E | x * (inv X) → X |
| 6623 | X ←→ Y |
| 662B | trn X → X |
| 6630 | det X → x |
| 6635 | -X → X |
| 663A | sgn X → X |
| 663F | X → M |
| 6647 | M → X |
| 664C | X + M → X |

X, Y and M are matrices (DIMensioned 2-dimensional BASIC arrays), while x (lower case here) is the simple BASIC variable X.

For example, suppose that you want to solve the following system of linear equations from the PC-1403 users manual (page 77):

2x + 5y -  z = -1

x -  y + 4z = 12

3x + 2y +  z =  9

The following program can do this (completes in about 2 seconds):

5 "A": CLEAR: RESTORE 20

10 DIM X(2,2): FOR I=0 TO 2: FOR J=0 TO 2: READ X(I,J): NEXT J: NEXT I

20 DATA 2,5,-1,1,-1,4,3,2,1

30 DIM Y(2,0): FOR I=0 TO 2: READ Y(I,0): NEXT I

40 DATA -1,12,9

50 CALL &6604: REM invX->X

60 CALL &65F7: REM X*Y->X

70 FOR I=0 TO 2: PRINT X(I,0): NEXT I

NOTE (as per PC-1403 Manual, page 44):

... matrixes X, Y, and M are stored in the same memory area as BASIC arrays X(*,*), Y(*.*), and M(*.*).

In other words, the values of the matrix elements entered in the BASIC mode can be calculated in the CAL mode.

When entering the values of matrix elements in the BASIC mode, pay attention to the following points:

(1) Matrix elements X(i, k) correspond to BASIC array elements X(i-1, k-1).

For example, X(1, 2) correspond to array X(0, 1).

(2) All the matrix element values stored in memory will be cleared by BASIC command RUN, CLEAR, or NEW.

## The 11-pin connector

Pinout:

| pin | Function |
| --- | --- |
| 1 (TOP) | MTout2 |
| 2 | VCC |
| 3 | GND |
| 4 | BUSY |
| 5 | Dout |
| 6 | Xin (MTin) |
| 7 | Xout (MTout1) |
| 8 | Din |
| 9 | ACK |
| 10 | I01 |
| 11 | I02 |

This is the internals schematics:

Using the PC-1403, output control works as follows.

The Pins 5,8,10,11 are connected to the external Ports 1-4 (Chip LZ92K32) for data output.

POKE 14848,1 sets Ext1 (Pin 11) to HIGH

POKE 14848,2 sets Ext2 (Pin 10) to HIGH

POKE 14848,4 sets Ext3 (Pin 5) to HIGH

POKE 14848,8 sets Ext4 (Pin 8) to HIGH

in machine language it’s the following code:

LIDP 14848

LIA x

STD

RTN

(16,58,0,2,x,82,55)

Xout, pin 7, also connected to the internal beeper and used for data-cassette writing, is controlled by the ‘Cout’ port on bit #4 (counting from 0), set with ‘AND 0x10’ for example with a machine program like below:

12 5F  LIP  0x5F        ; 0x5F --> P

61 10  ORIM 0x10        ; (P) | 0x10 --> (P)

DF     OUTC             ; (0x5F) --> Port C

or reset with:

12 5F  LIP  0x5F        ; 0x5F --> P

60 10  ANIM 0x00        ; (P) & 0x00 --> (P)

DF     OUTC             ; (0x5F) --> Port C

## CE 140 F (Disk drive)

(Excerpts from the CE-140 Service Manual)

DATA TRANSFER

o Data transmit protocol (pocket computer to FDD)

The pocket computer sets low order four bits of one byte data on DIN, DOUT, I02 and I01. Then, BUSY is forced high. On the FDD side, ACK is forced low after confirming a low state of BUSY to finish the first handshaking. As the second handshaking is done in the same manner, transfer of one byte data is completed.

o Data receive protocol (FDD to pocket computer)

If ACK is high for the FDD is in process, ACK is set low for more than 10ms before data are transferred from the FDD after completion of its processing.

The FDD set ACK high after setting data on DIN, DOUT, IO2 and IO1. However, the data must be sent out in 50ms after ACK turned low.

After recognizing a high state of ACK, the pocket computer receives a 4-bit data, then BUSY is set low. After the FDD confirms a high state of BUSY, ACK is set low. After the pocket computer confirms a low state of ACK, BUSY is forced low to terminate the first handshaking. A single byte data is received by repeating this procedure.

## PC-1403 Schematics